Portable STM32-Based Cardiac-Fall Fusion Monitor

Using On-Device TinyML ECG Analysis

Project Definition Report: Problem Statement, Research Gap, Objectives, Component Finalization & Bi-Weekly Work Plan

Group No: ETC_10  |  School of Electronics Engineering, KIIT Deemed to be University

Project Supervisor: Dr. Nirmal Kumar Rout

Members: Arjoyeeta Chakraborty (2304014)  ·  Prince Kumar (2304105)  ·  Sadhika Das (2304113)

7th Semester (August – mid-November 2026) and 8th Semester, B.Tech Major Project

1. Background

Cardiovascular emergencies become significantly more dangerous when they occur alongside a fall or loss of consciousness, a risk that is especially high for elderly individuals or people living alone. Existing options outside a hospital setting are limited to periodic checkups, wearables that log data for later review, or fitness trackers that measure heart rate without interpreting whether the underlying rhythm is actually abnormal. This project sets out to close that gap with a portable, battery-powered device that reads a person's ECG in real time, analyzes it on-device using a lightweight AI (TinyML) model running entirely on an STM32 microcontroller, and fuses this with fall detection so that a rhythm abnormality and a fall occurring together are flagged as a single, higher-priority emergency.

2. Problem Statement (Final — 7th & 8th Semester)

Sudden cardiac events are considerably more life-threatening when they coincide with a fall, yet no accessible, low-cost system currently recognizes this combination as a single emergency. Reviewed prior work shows that each existing approach solves only part of the problem: on-device AI arrhythmia detectors are validated only on offline, pre-recorded datasets rather than a live sensor signal and cannot classify the type of abnormality; fall-detection wearables track basic heart rate rather than the true ECG waveform and rely on remote servers rather than on-device intelligence; and systems that do capture and analyze the real ECG waveform typically depend on a separate connected computer and ignore falls entirely. No existing low-cost, portable system combines real-time ECG acquisition, on-device (edge) AI rhythm analysis, and fall detection into one embedded device capable of raising a single, escalated alert when a cardiac abnormality and a fall happen close together in time.

This project addresses that gap directly: designing and building a portable, battery-operated embedded device — built around an STM32 microcontroller — that continuously acquires a person's ECG through chest electrodes, performs digital filtering and R-peak/heart-rate extraction, runs a TinyML model on-device to classify the rhythm as normal or abnormal, and monitors SpO₂ and body temperature as supporting vital signs, all fused with real-time fall detection from an accelerometer/gyroscope to raise a local, escalated alert when a cardiac or vital-sign abnormality and a fall occur together — with no dependency on the internet, cloud processing, or a paired computer for detection. This problem statement is scoped to be fully addressed using only locally-detected signals (ECG, SpO₂, temperature, and motion) within the 7th and 8th semester timeline; a cellular/GPS-based remote-notification layer is treated as an optional extension (Section 4) rather than part of the core problem being solved.

3. Research Gap

Across the systems reviewed for this project, the following recurring gaps stand out:

- On-device AI arrhythmia classifiers are demonstrated on static, pre-recorded ECG datasets (e.g., MIT-BIH) rather than on a live sensor reading an actual heartbeat in real time, and typically only flag a rhythm as abnormal without further classification.

- Systems that combine heart monitoring with fall detection generally track basic heart rate (via PPG/pulse) rather than the true ECG electrical signal, and run their intelligence on a remote server or smartphone app rather than on the embedded device itself, introducing latency and a dependency on connectivity.

- Systems that do read and analyze the real ECG waveform in detail are usually tethered to a separate connected computer for processing and do not track falls at all — so a cardiac event and a fall are never recognized as a compounded, higher-priority emergency.

- Very few portable, low-cost embedded systems integrate ECG, SpO₂, and body temperature alongside fall detection into a single device — most commercial and research prototypes cover only one or two of these vitals, leaving multi-parameter emergency correlation largely unaddressed.

- Continuous, fully offline (no-cloud) AI-based monitoring for low-cost, home-use embedded devices remains an active and underexplored research area, particularly on resource-constrained microcontrollers such as the STM32 family.

No system found in this review combines live ECG acquisition, on-device TinyML rhythm analysis, SpO₂/temperature monitoring, and real-time fall detection with a fusion/escalation mechanism — this is the specific, well-defined gap this project targets.

4. Aims and Objectives

Aim: To design, build, and validate a portable, battery-powered embedded device that performs real-time ECG acquisition and on-device TinyML rhythm classification, monitors SpO₂ and body temperature as supporting vitals, and detects falls — with a fusion mechanism that escalates alerts when a cardiac/vital-sign abnormality and a fall occur together — entirely offline, on an STM32 microcontroller.

The objectives below are split into what is realistically achievable in the 7th semester (Aug–mid Nov 2026) and what carries into the 8th semester, so the same objective set holds as the final scope for the whole project.

4.1 Core Objectives — 7th Semester (Aug – mid-Nov 2026)

- Acquire the ECG signal through chest electrodes using the AD8232 analog front-end, and interface it with the STM32's ADC.

- Implement digital signal processing on-device: noise/baseline-wander removal, band-pass/notch filtering, R-peak detection, and real-time heart-rate calculation.

- Integrate the MPU6050 accelerometer/gyroscope and implement a motion-based fall-detection algorithm.

- Integrate the MAX30102 SpO₂/pulse sensor and a DS18B20 temperature sensor, and acquire continuous SpO₂ and body-temperature readings alongside ECG.

- Build the supporting hardware: OLED status display (showing ECG-derived heart rate, SpO₂, and temperature), buzzer/LED local alarm, push-button reset, and battery + charging circuit (Li-ion 18650 + TP4056).

- Collect/prepare ECG data (MIT-BIH Arrhythmia Database) and begin TinyML model design for normal/abnormal rhythm classification.

- Deliver a working breadboard-level prototype demonstrating live ECG display, heart-rate calculation, and fall detection as independent modules, ready for AI integration.

4.2 Core Objectives — 8th Semester

- Train, quantize (INT8), and deploy the TinyML model on-device using TensorFlow Lite Micro for real-time normal/abnormal rhythm classification.

- Design and implement the fusion/escalation logic that raises a higher-priority alert when an abnormal rhythm, an out-of-range SpO₂/temperature reading, and/or a fall are detected within a defined time window of each other.

- Integrate all modules (ECG + AI + SpO₂ + temperature + fall detection + display + alarm + power) into a single, enclosed, battery-operated prototype.

- Evaluate system performance: classification accuracy, real-time response latency, false-alarm rate, and battery life/power consumption, benchmarked against reviewed published work.

- Move from breadboard to a compact custom/perfboard layout with attention to ECG analog signal routing, grounding, and noise isolation.

- Compile the final report, literature survey, and prepare the project for evaluation/demonstration.

4.3 Extension Objective — If Time Permits (8th Semester, Stretch Goal)

The literature and component-comparison work also explored GPS-based location and GSM/cellular-based remote emergency alerting (e.g., NEO-6M GPS + SIM800L GSM). This capability is not part of the core problem statement above — it is retained explicitly as an optional extension, to be attempted only after the core ECG + TinyML + fall-fusion system is fully working and evaluated:

- Add a NEO-6M GPS module to capture the patient's location at the moment of an escalated alert.

- Add a SIM800L GSM module to send an automatic SMS (and/or trigger a call) with the location and alert type to a pre-configured emergency contact, using a spare STM32 UART.

- This extension turns the device from a local-alarm prototype into a remote-notification-capable one, but is explicitly scoped as a stretch goal so the core deliverable is never put at risk by it.

5. Microcontroller Board Selection

The project needs one board to handle ECG acquisition, digital filtering, R-peak/BPM calculation, SpO₂ + temperature + fall-detection sensor reads, OLED output, and an on-device TinyML rhythm classifier — with GSM added as an 8th-sem extension only if time permits. The comparison below is drawn from the processor-board comparison research, narrowed to the boards that are realistic for a 3-member, cost-conscious semester project.

| Board | CPU / RAM | TinyML headroom | Price (₹) | Verdict for this project |
| --- | --- | --- | --- | --- |
| STM32F401RE Nucleo | 84 MHz M4F / 96 KB RAM | Limited — workable but tight for the model + DSP buffers together | ~1,800–1,950 | Sufficient, but the least headroom of the realistic options |
| STM32F446RE Nucleo | 180 MHz M4F / 128 KB RAM | Good for small/medium models — over 2× the clock and 33% more RAM than F401RE | ~1,800–1,950 | Best fit: same price as F401RE, clearly more capable |
| STM32G474RE Nucleo | 170 MHz M4 / 128 KB RAM | Good for small models; strongest for control/DSP-heavy work | ~1,900–3,185 | Comparable capability to F446RE but costs more in most listings |
| STM32H753ZI Nucleo | 480 MHz M7 / 1 MB RAM | Excellent — most headroom of all options | ~3,000–5,000 | Overkill for the scope and cost of this project |

Verdict: the STM32F446RE Nucleo is the best-suited board for this project. It sits at essentially the same price as the F401RE in every India listing checked (₹1,798 DigiKey, ₹2,249 Hubtronics), but runs at 180 MHz versus 84 MHz and has 128 KB of RAM versus 96 KB — giving noticeably more comfortable headroom for running the ECG DSP pipeline and the quantized TinyML model side by side, without adding any cost or changing the toolchain (same STM32CubeIDE/HAL workflow, same Nucleo-64 form factor, same Arduino/Morpho headers). The G474RE offers similar capability but is not consistently cheaper, and the H753ZI's extra headroom isn't needed at this project's scope. The component list below has been updated to the F446RE.

6. Finalized Component List (Lowest-Cost Option Meeting Requirements)

Components below are selected to meet the project's functional requirements at the lowest practical cost, based on current India listings. Prices are seller snapshots and can vary with stock, quantity, and shipping — confirm before ordering.

6.1 Core System (Required)

| Component | Selected Part | Why this one | Approx. Price (₹) | Where to buy |
| --- | --- | --- | --- | --- |
| Microcontroller | STM32 Nucleo-F446RE | Best-suited board per Section 5: same price as F401RE, but 180 MHz / 128 KB RAM gives comfortably more headroom for running ECG DSP and the TinyML model together. | ~1,800–1,950 | DigiKey / Hubtronics |
| ECG front-end | AD8232 ECG module | Lowest-cost, best-documented ECG analog front-end; simple ADC interfacing with STM32. | ~375–540 | Robocraze / Robu.in |
| ECG electrodes | Disposable Ag/AgCl electrode pack | Standard, cheap, needed for every test session. | ~150 | Robu.in |
| Fall detection | MPU6050 (GY-521 breakout) | Cheapest 6-axis accelerometer+gyroscope module; I²C, ample for fall/motion detection. | ~90–150 | Robokits / Robu.in |
| SpO₂ / pulse | MAX30102 | Very common, inexpensive I²C SpO₂ + pulse (PPG) module; shares the same I²C bus as the OLED and MPU6050. | ~115–230 | Robu.in / Robocraze |
| Body temperature | DS18B20 (digital, 1-Wire) | Cheapest reliable temperature sensor; probe versions make skin contact easy. (Upgrade option: MAX30205, ~₹1,000–1,500, purpose-built for human body temperature if budget allows.) | ~100–250 | Robu.in |
| Display | 0.96" SSD1306 OLED (I²C) | Cheapest legible display option; only 2 I/O pins needed (shares I²C bus with MPU6050 and MAX30102). | ~150–200 | Robu.in |
| Local alarm | Piezo buzzer + LED | Immediate local audible/visual alert. | ~30–50 | Any local electronics store / Robu.in |
| Manual trigger | SOS push button | User-triggered manual emergency path, redundant to auto-detection. | ~20–30 | Robu.in |
| Power | 18650 Li-ion cell + holder | Rechargeable, portable power for a wearable/handheld prototype. | ~250–400 | Robu.in |
| Charging | TP4056 Li-ion charging module | Cheapest safe way to recharge the 18650 cell via USB. | ~40–80 | Robu.in |
| Wiring/build | Breadboard, jumper wires, perfboard, enclosure | For breadboard stage (7th sem) then perfboard/enclosure integration (8th sem). | ~500–600 | Local electronics store |

Core system subtotal: ≈ ₹3,600 – ₹4,900 (before contingency)

6.2 Extension: GPS + GSM (Only If Time Permits, 8th Sem)

| Component | Selected Part | Why this one | Approx. Price (₹) | Where to buy |
| --- | --- | --- | --- | --- |
| GPS | NEO-6M GPS module | Cheapest common GPS module with adequate accuracy for emergency location. | ~239–480 | Robu.in |
| Cellular alert | SIM800L GSM/GPRS module | Cheapest way to send an SMS alert; note 2G availability should be checked in the deployment area (use Airtel/Vi/BSNL, not Jio, for 2G). | ~249–410 | Robu.in / Robocraze |

Extension subtotal: ≈ ₹500 – ₹900 additional, only if pursued after the core system is complete

6.3 Component Links

STM32 Nucleo-F446RE — DigiKey India

STM32 Nucleo-F446RE — Hubtronics

AD8232 ECG Module — Robocraze

MPU6050 (GY-521) — Robokits India

MPU6050 — Robu.in

0.96" SSD1306 OLED I²C — Robu.in

MAX30102 SpO₂/Pulse Sensor — Robu.in

DS18B20 Waterproof Temperature Probe — Robu.in

NEO-6M GPS Module — Robu.in

SIM800L GSM/GPRS Module — Robu.in

Note: AD8232/MAX30102-class modules are engineering/research-grade, not medical-grade devices — the prototype should be described and evaluated as a research/engineering device.

7. Bi-Weekly Work Distribution Plan

Work starts from the 1st week of September 2026 and runs through the 7th semester (ending mid-November 2026) into the 8th semester. Roles are realigned as follows: Sadhika and Prince jointly cover digital signal processing, hardware components/sourcing, dataset preparation, and all hardware integration (split between the two of them below); Arjoyeeta focuses on the TinyML model — design, training, quantization, and on-device deployment — and additionally takes the GSM extension in the 8th semester if time permits.

7.1 Seventh Semester (1 Sep – mid-Nov 2026)

| Period | Sadhika Das | Prince Kumar | Arjoyeeta Chakraborty |
| --- | --- | --- | --- |
| 1–15 Sep | Set up AD8232–STM32 ADC interfacing on the Nucleo-F446RE; capture raw ECG; begin digital filtering (baseline wander + notch). (algorithm Design) | Source all core components (Section 6.1); set up breadboard and power rails; download and organize the MIT-BIH Arrhythmia Database. | Study the organized MIT-BIH dataset structure; define the TinyML model architecture (lightweight 1D-CNN) suited to the F446RE's memory budget. |
| 16–30 Sep | Implement band-pass filtering, R-peak detection, and real-time BPM calculation; integrate MPU6050 for fall detection. | Bring up the OLED display over I²C; integrate MAX30102 (SpO₂) and DS18B20 (temperature) on the shared bus; verify no address conflicts. | Preprocess and label the ECG dataset for normal/abnormal segmentation; begin baseline model training in TensorFlow/Keras. |
| 1–15 Oct | Develop and tune the fall-detection algorithm (threshold + orientation change) to reduce false positives. | Integrate buzzer, LED, and SOS push button; wire the battery (18650) + TP4056 charging circuit; measure baseline power draw. | Continue model training/iteration; evaluate accuracy, precision, and recall on held-out dataset splits. |
| 16–31 Oct | Draft the fusion/escalation logic design (how abnormal rhythm, out-of-range SpO₂/temperature, and a fall within a time window are combined). | Consolidate all modules on the breadboard into one connected prototype; run an end-to-end smoke test. | Begin INT8 quantization and conversion to TensorFlow Lite Micro format; bench-test the quantized model on a PC/emulator against sample ECG windows. |
| 1–15 Nov | Document the DSP, fall-detection, and fusion-logic design for the 7th-sem report; prepare a demo of ECG, SpO₂/temperature, and fall detection running independently. | Document component sourcing and hardware integration for the 7th-sem report; lead compilation of the 7th-sem report and end-semester demo. | Document the TinyML pipeline, training results, and preliminary quantization outcome for the 7th-sem report. |

7.2 Eighth Semester

Exact bi-weekly calendar dates depend on the official 8th-semester academic calendar (not yet published); the phases below are sequenced to slot into that calendar's fortnightly periods once released.

| Phase (≈2 weeks each) | Sadhika Das | Prince Kumar | Arjoyeeta Chakraborty |
| --- | --- | --- | --- |
| Phase 1 | Support integration of the quantized model into the STM32 firmware alongside the DSP pipeline. | Move the build from breadboard to perfboard; improve ECG analog grounding/routing and sensor wiring. | Deploy the TensorFlow Lite Micro model on-device; validate on live ECG rather than only offline data. |
| Phase 2 | Implement and test the fusion/escalation logic on-device (abnormal rhythm + SpO₂/temperature out-of-range + fall → escalated alert). | Finalize enclosure design; mount all modules into a portable form factor. | Tune model performance on-device (latency, memory footprint, accuracy) after deployment. |
| Phase 3 | Run integrated end-to-end tests: real ECG + SpO₂/temperature + simulated falls; verify correct escalation behavior. | Measure and optimize battery life under realistic usage; document the power budget. | Run accuracy/latency benchmarking against reviewed published systems for the report. |
| Phase 4 (Extension, if time permits) | Support hardware wiring/trigger logic for when a GPS+SMS alert should fire, alongside Prince. | Integrate NEO-6M + SIM800L hardware and UART wiring, alongside Sadhika, if the extension is pursued. | Own the GSM software layer: AT-command interfacing, message-content logic (alert type + coordinates), and testing SMS delivery, if time permits. |
| Final Phase | Compile DSP/fusion-logic documentation; contribute to the final report and literature survey. | Lead final report compilation, demonstration rehearsal, and viva/evaluation preparation. | Compile TinyML methodology, results, and (if pursued) GSM extension documentation for the final report. |

8. Summary

- Problem statement and research gap are finalized to cover the full project (7th + 8th semester) — see Sections 2–3.

- Core objectives (ECG + on-device TinyML + SpO₂ + temperature + fall fusion) are fully achievable within the 7th + 8th semester timeline without GPS/GSM.

- GPS + GSM/cellular alerting is explicitly scoped as an 8th-semester stretch goal, owned by Arjoyeeta on the software side, only pursued once the core system is complete (Section 4.3, 6.2).

- The Nucleo-F446RE replaces the F401RE as the selected board (Section 5): same price, but more headroom to run ECG DSP and the TinyML model together.

- Finalized core component set (ECG + SpO₂ + temperature + fall detection) costs approximately ₹3,600–₹4,900; the GPS/GSM extension adds approximately ₹500–₹900 more.

- Work is divided bi-weekly starting 1 September 2026: Sadhika and Prince split DSP, hardware components/sourcing, dataset preparation, and hardware integration between them; Arjoyeeta owns the TinyML model (design through on-device deployment) and the GSM extension if time permits.